ET AI HACKATHON 2026  ·  PROBLEM STATEMENT 3

VoltEdge AI

AI for Industrial EV Supply Chain & Asset Intelligence

An intelligence layer over the four asset domains an industrial EV operator already has data for — battery health, fleet electrification, multi-tier supply chain, and manufacturing quality — joined by one rule: every figure the system reports traces back to the tool call that produced it.

Project report  ·  Working prototype, architecture, measured evidence and limitations

# Contents

# 1. Executive summary

VoltEdge AI is a working prototype that addresses industrial EV adoption from the two angles the problem statement names: helping asset-intensive organisations run EV fleets with the rigour they apply to conventional equipment, and helping manufacturers manage quality-critical battery supply chains.

The prototype is complete and running. It comprises a FastAPI backend exposing 41 endpoints across 8 routers, a Next.js application with 8 interactive pages, seven domain services, a five-agent orchestration layer over a ten-tool registry, and a cross-module compound risk engine.

## What distinguishes it

Most fleet software presents four separate dashboards. The decisions that actually cause loss fall in the gaps between them: a battery ages normally, a cobalt alert fires, a production line stays in spec — each individually unalarming, and nobody connects them.

VoltEdge is built around that gap. Its compound risk engine correlates signals across modules and, for every finding, states explicitly which modules it joined and why no single module would have escalated on its own.

## Headline results

| Claim | Measured | Baseline | Data |
| --- | --- | --- | --- |
| Battery degradation forecast (RMSE, %SoH) | 2.281 | 3.718 persistence | Real — NASA PCoE |
| Remaining useful life error | 16 cycles | 10.6% of cell life | Real — ground-truth EOL |
| Manufacturing defect recall | 0.854 | 0.521 (3σ SPC) | Synthetic, 2,600 batches |
| Defect detection ROC-AUC | 0.903 | at 17.7% flagged | Synthetic, hold-out |
| Process drift detection | 4.9σ | caught while in spec | Synthetic (blade wear) |
| Fleet carbon saved vs all-diesel | 64.6 t/yr | 8.8% reduction | Modelled, assumptions in API |

Evidence posture Every performance claim is validated on held-out data against a stated baseline, and every figure is labelled by provenance: real, synthetic, or modelled. Where real data exists it is used; where it does not, the document and the application both say so on screen.

# 2. Problem context

India registered over two million electric vehicles in FY2025, yet EVs remain under 7% of total vehicle sales. In industrial and commercial segments — freight, mining, intra-plant logistics, construction equipment — penetration is below 2.5% per SIAM FY2025 data.

The barrier is no longer primarily financial. FAME-II has disbursed over ₹10,000 crore, and total-cost-of-ownership parity with diesel is approaching for many use cases. The bottleneck is operational: industrial fleet operators lack the asset intelligence tooling to manage EV procurement, battery lifecycle, charging infrastructure and maintenance with the rigour they apply to conventional plant.

Simultaneously, EV manufacturers face supply chain complexity that generic tools handle poorly — battery-grade lithium, cobalt and nickel sourcing, cell-to-pack quality traceability, and multi-tier supplier risk.

## The insight we built on

The claim that the barrier is "operational" is a claim about fragmentation, not about missing data. The data largely exists. What is missing is anything that reasons across it.

The failure mode is not a missing metric. It is a missing correlation.

# 3. Solution overview

VoltEdge unifies four domains behind a single intelligence layer, with the battery passport acting as the connective tissue that lets a signal in one domain resolve to specific assets in another.

| Module | Route | What it answers |
| --- | --- | --- |
| Command Center | / | Fleet-wide position across all four domains |
| Agent Console | /agent-console | Cross-module questions; compound risk findings |
| Battery APM | /battery-apm | State of health, degradation forecasts, thermal alerts |
| Battery Passport | /battery-passport | Pack identity, AIS-156 compliance, second-life value |
| Quality (QMS) | /quality | Defect detection vs SPC, process drift, traceability |
| Fleet Readiness | /fleet-readiness | Which diesel vehicles to electrify, and the TCO case |
| Supply Chain Risk | /supply-chain | Multi-tier supplier and critical-mineral exposure |
| Net Zero | /net-zero | Scope 1/2/3 emissions and highest-impact next actions |

## The connective tissue

The Battery Pack Aadhaar — a 21-character pack identity carrying immutable manufacturing facts and a live operational ledger — is what turns four modules into one system.

- A cobalt supply alert resolves to specific vehicles, because the pack's material composition record says which chemistry it uses.

- A flagged manufacturing batch resolves to specific vehicles on the road, through the cell → pack → vehicle lineage.

- An ageing pack resolves to a procurement decision, because its chemistry determines its replacement lead time.

# 4. System architecture

The system is a five-layer pipeline. Signal enters at the data layer and leaves as a decision at the experience layer.

| Layer | Contents |
| --- | --- |
| 01 · Data | NASA PCoE cell ageing data (real); CEA grid factors (published); BMS telemetry, supplier registry and production line records (modelled) |
| 02 · Domain services | degradation.py, qms.py, bpan.py, compound.py, nasa_data.py, agents.py, seed_data.py — deterministic and independently testable |
| 03 · Intelligence | Planner → 5 specialist agents → synthesiser, over 10 deterministic tools; compound risk engine correlating 4 modules |
| 04 · API | FastAPI — 41 endpoints across 8 routers, browsable at /docs |
| 05 · Experience | Next.js 16 — 8 pages, every asset clickable through to its underlying evidence |

## The architectural guarantee

Tools are the source of truth; the model only reasons Every number a specialist agent states came from a deterministic read against the live data store. The language model interprets figures it was handed — it never generates them. This keeps the platform auditable, which is a hard requirement for anything touching battery safety or regulatory reporting. It also means the orchestrator degrades gracefully: with no model key configured it falls back to deterministic planning and template synthesis, so a demonstration never depends on an external API.

## Agent roster

| Agent | Remit |
| --- | --- |
| Battery APM | Predictive battery health, degradation physics, remaining useful life |
| Supply Chain Risk | Multi-tier supplier risk, critical minerals, disruption monitoring |
| Procurement & Readiness | Electrification readiness scoring, TCO, procurement planning |
| Carbon Intelligence | Scope 1/2/3 accounting, BRSR Core reporting, decarbonisation priorities |
| Compliance & Traceability | Battery Pack Aadhaar traceability, AIS-156 Phase 2 safety |

# 5. Module detail

## 5.1  Battery Asset Performance Management

Monitors state of health, charging patterns, thermal events and degradation trajectories across the EV fleet, producing remaining-useful-life estimates and predictive maintenance triggers.

### The degradation model

The model form is physics-informed rather than a black box: SOH(t) = 100 − a·√t − b·t

- The √t term captures calendar ageing driven by solid-electrolyte interphase (SEI) layer growth, which follows a square-root-of-time law.

- The linear term captures cycling ageing — loss of active material, approximately linear in throughput.

Constraining the functional form to two coefficients keeps the fit stable on short, noisy per-asset histories and avoids the out-of-distribution collapse that affects unconstrained regressors on battery data.

### Per-asset model selection

Cells entering the capacity "knee" — accelerated non-linear fade near end of life — are poorly described by their own early history. The model therefore selects an exponential recency weighting per asset, chosen on an inner split of the training data so the held-out test set is never consulted. This lifted real-data improvement over baseline from 20.2% to 38.6%.

## 5.2  Manufacturing Quality Intelligence (QMS)

Two classifiers are trained deliberately, because the brief asks for defects to be caught before product reaches assembly.

- Early warning — process parameters and incoming material only. Known the moment a batch is coated and welded, hours before in-line inspection returns a verdict. This is the model that buys lead time.

- Full — adds in-line inspection results. By the time these exist, the cells are already built.

Both are compared against the rule a conventional QMS actually runs: 3-sigma control limits on individual process parameters.

### Feature engineering

Defect risk depends on absolute deviation from target — a coating that is too thin is as damaging as one too thick. Features are therefore expressed as |value − target| ÷ tolerance. Feeding raw process values to a linear model cannot represent that U-shaped relationship and scored near random (measured ROC-AUC 0.45). This was the single most consequential correctness issue in the module.

### Additional capability

- Process drift detection — catches a parameter walking off target while individual batches remain in spec, so no reject has yet been raised.

- SPC control limits with process capability (Cpk) against the automotive 1.33 bar.

- Per-batch driver attribution for root-cause analysis.

- Cell → pack → vehicle traceability linking a flagged batch to fielded assets.

## 5.3  Battery Pack Aadhaar (BPAN)

Implements the identity and data architecture of MoRTH's draft Battery Pack Aadhaar scheme: a 21-character checksummed identity per pack, split into an immutable static schema and a dynamic operational ledger.

| Schema | Contents |
| --- | --- |
| BMI — Manufacturer identifier | Country of origin, factory code, manufacture date |
| BDS — Battery descriptor | Chemistry, nominal capacity, nominal voltage, vehicle model |
| BMCS — Material composition | Critical mineral percentages; drives supply-chain exposure mapping |
| BCF — Carbon footprint | Embedded manufacturing emissions, cradle-to-gate |
| BDD — Dynamic ledger | SoH, cycles, RUL, AIS-156 safety findings, lifecycle status |

The module also performs an AIS-156 Phase 2 safety audit from BMS telemetry and computes second-life routing and residual value for packs past traction duty.

## 5.4  Fleet Electrification Readiness

Scores every diesel vehicle on an Electrification Readiness Index weighing route compatibility, depot dwell time, total cost of ownership, infrastructure readiness and operational criticality, then matches each to a viable EV replacement.

Matching enforces both range and payload, and rejects gross over-specification: a candidate more than 2.5× the required payload is not a real recommendation, because putting a four-tonne truck under a one-tonne duty cycle destroys the TCO case. Where no vehicle qualifies, the system reports that honestly rather than proposing an implausible swap.

## 5.5  Supply Chain Risk & Traceability

Tracks 15 suppliers across five tiers from mining through refining, component manufacture and cell production to pack assembly, scoring each on geopolitical exposure, quality, concentration, ESG and financial stability.

## 5.6  Net Zero & Carbon Intelligence

Computes Scope 1, 2 and 3 emissions for the mixed fleet against a counterfactual all-diesel operation, with per-route carbon intensity and a ranked list of highest-impact electrification actions.

Assumptions are exposed through the API rather than buried in code: grid_factor_kg_per_kwh (CEA 0.71) and renewable_depot_share_pct (30%) are returned in the summary response so a reviewer can inspect and challenge them.

## 5.7  Compound Risk Engine

The cross-module layer. Every signal it uses is individually visible in some existing module and individually unalarming; the value is in the correlation. Each finding carries an explicit evidence chain naming the contributing module, so a reviewer can audit why the platform escalated.

| Finding | Severity | Confidence |
| --- | --- | --- |
| NMC supply disruption collides with pack replacement window | High | 0.86 |
| Fast-charging habit is driving the thermal-event cluster | High | 0.81 |
| LFP supply disruption collides with pack replacement window | High | 0.72 |
| Single-geography concentration carries an embedded-carbon penalty | Medium | 0.74 |

# 6. Worked example: the compound signal path

This is the platform's signature capability — four modules producing one escalation that none of them would raise alone.

| Step | Source module | Signal |
| --- | --- | --- |
| 1 | supply_chain.alerts | DRC cobalt export restriction; spot price up 18% this week |
| 2 | supply_chain.suppliers | Tenke Fungurume Mining, Tier 4, composite risk 59.1 |
| 3 | bpan.bmcs | Material composition identifies which packs are NMC chemistry |
| 4 | battery_apm.rul | Earliest exposed pack at 78.4% SoH — 11 months of runway |
| 5 | procurement | NMC replacement lead time is 18 weeks |

Why no single module raises this Supply chain sees a critical material alert. Battery APM sees packs ageing normally. Neither connects the disruption to the replacement window — which is still open, but only just. Recommended actions generated: raise the NMC replacement purchase order now; dual-source by qualifying an India-localised cell supplier to cut lead time; pivot new procurement to LFP, which removes cobalt exposure entirely and shortens lead time to 12 weeks.

# 7. Data provenance

This section matters more than any individual number, and the application labels provenance on screen wherever a figure appears.

## 7.1  Real data — battery degradation

Battery degradation is validated against real measured cells from the NASA Prognostics Center of Excellence Li-ion ageing dataset (CC0, public domain), retrieved via a community mirror because NASA's own portal no longer serves the files. It should be cited as "NASA PCoE dataset (community mirror)", not as a direct NASA feed.

| Cell | Cycles | Initial capacity | Final capacity | Total fade | Mean temp |
| --- | --- | --- | --- | --- | --- |
| B0005 | 168 | 1.856 Ah | 1.325 Ah | 28.6% | 39.8 °C |
| B0006 | 168 | 2.035 Ah | 1.186 Ah | 41.7% | 39.9 °C |
| B0007 | 168 | 1.891 Ah | 1.432 Ah | 24.3% | 40.4 °C |
| B0018 | 132 | 1.855 Ah | 1.341 Ah | 27.7% | 37.7 °C |

### Cells deliberately excluded

The mirror also carries cells B0049–B0056. These were excluded after inspection, for cause:

- B0050 terminates at a measured capacity of 0.000 Ah — truncated or corrupt.

- B0051, B0054, B0055 and B0056 show capacity increasing across the test (−5% to −44% "fade"). These are low-ambient protocols where measured capacity recovers; they are not monotonic degradation curves and would corrupt the validation.

- B0053 shows only 5.5% fade over 55 cycles — too little signal to validate an end-of-life forecast against.

The synthetic-versus-real gap, stated plainly The same degradation model scores 0.475 %SoH RMSE on our synthetic fleet but 2.281 on real cells. That gap is the point: fitting data generated by a process we wrote ourselves is partly circular, and the flattering number proves little. The real-cell figure is the honest measure, and it is what the application leads with.

## 7.2  Synthetic data — manufacturing quality

Gigafactories do not publish defect data, and there is no open cell-manufacturing defect corpus that could be vendored the way the NASA data was. Precision and recall for the QMS module are therefore measured on generated batches with a known causal structure, and the Quality page states this on screen.

What transfers to real production data is the pipeline: the feature construction, the SPC baseline it is compared against, the stratified hold-out evaluation, and the safety-weighted operating point.

## 7.3  Modelled data — carbon and fleet

Carbon figures are modelled from published emission factors, with assumptions exposed via the API. Fleet telemetry, supplier registry and vehicle records are synthetic but grounded in real Indian market data — actual EV and diesel models, real OEMs, real city grid intensities.

## 7.4  Regulatory status

The Battery Pack Aadhaar is a draft MoRTH scheme. The BPAN module is built for compliance-readiness and must not be presented as a live regulatory integration.

# 8. Measured evidence

## 8.1  Battery degradation — real cells

Validation is a hold-out on the last 30% of each cell's cycle history, against a persistence baseline (last observed SoH carried forward), which is the standard naive forecaster for monotonic series.

| Cell | Model RMSE | Baseline RMSE | Improvement | Weighting selected |
| --- | --- | --- | --- | --- |
| B0005 | 2.089 | 3.769 | +44.6% | recency 1/2 |
| B0006 | 1.954 | 5.502 | +64.5% | recency 1/5 |
| B0007 | 1.512 | 3.210 | +52.9% | unweighted |
| B0018 | 3.570 | 2.391 | −49.3% | recency 1/5 |
| Mean | 2.281 | 3.718 | +38.6% | 3 of 4 beat baseline |

Remaining useful life is forecast from half of each cell's cycle life — not from the 70% training split, where only a handful of cycles of true runway remain and percentage error explodes on a tiny denominator. Mean absolute error is 16 cycles, or 10.6% of total cell life, across the three cells that reach the end-of-life threshold.

B0018 is reported as a miss. The model underperforms the baseline on that cell. It is included rather than excluded because selectively reporting only favourable cells would misrepresent the result.

## 8.2  Manufacturing defect detection

Stratified 70/30 hold-out on 2,600 statistically stable batches, at a 6.1% defect rate. The classifier is trained only on stable production; the drifting tail is held out of it and handled by the drift monitor instead, because fitting a quality model on out-of-control production is a methodology error.

| Approach | Precision | Recall | F1 | AUC | Flagged | Caught | Missed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Early warning (process + material) | 0.297 | 0.854 | 0.441 | 0.903 | 17.7% | 41 | 7 |
| Full (adds in-line inspection) | 0.297 | 0.854 | 0.441 | 0.901 | 17.7% | 41 | 7 |
| SPC baseline (3σ, single parameter) | 0.275 | 0.521 | 0.360 | — | 11.7% | 25 | 23 |

### Reading these numbers

Precision of 0.297 is low in isolation, and deliberately so. Defects are rare at 6.1%, and the operating point targets recall because a missed escape reaching pack assembly costs far more than re-inspecting a good batch. The correct framing is screening: re-inspect 17.7% of production and catch 85% of escapes, against SPC catching 52% for 11.7%.

The full model, which adds in-line inspection, performs identically to the early-warning model at this operating point and marginally worse on AUC. The inspection signals are themselves consequences of the same process excursions, so they carry almost no independent information — which strengthens rather than weakens the case for acting on process data, because process data arrives first.

## 8.3  Process drift

Coating thickness has moved 2.31 µm from baseline over the last 120 batches — a 4.9σ shift — while remaining inside the engineering specification, so no reject has been raised and conventional SPC is silent. This is the case drift detection exists to catch.

## 8.4  Fleet and carbon position

| Metric | Value |
| --- | --- |
| Fleet composition | 50 vehicles — 20 EV, 30 diesel, 40% electrified |
| Average fleet state of health | 88.5% (1 critical, 4 warning) |
| Diesel vehicles ready to electrify now | 2 of 30 (ERI ≥ 80) |
| Diesel vehicles with no viable EV | 21 of 30 |
| Annual emissions | 667.2 t CO₂e (Scope 1 58.5%, Scope 2 23.5%, Scope 3 18.0%) |
| Carbon saved vs all-diesel | 64.6 t/yr (8.8%) |
| Packs traced with battery passport | 20 of 20 (100% coverage) |
| Packs flagged under AIS-156 | 5 of 20 |
| Second-life candidates | 3 packs, 235.2 kWh, ₹9.41 lakh residual value |
| Suppliers tracked | 15 across 5 tiers; 1 critical, 7 high risk |

# 9. Notable findings

Three results are worth stating plainly, because they appear to be weaknesses and are not.

## 9.1  Most of the fleet cannot be electrified today

21 of 30 diesel vehicles have no viable EV replacement. Payload and range matching rejects gross over-specification, and medium and heavy e-trucks fail their seven-year TCO case at current capital cost — a ₹55 lakh e-truck replacing an ₹18 lakh diesel does not recover the capital gap through fuel savings alone within the horizon.

This is precisely why industrial EV penetration sits below 2.5%. A tool that conceals it by recommending a three-wheeler as a replacement for a ten-tonne truck is not being helpful. The honest finding is more valuable than a flattering one.

## 9.2  In-line inspection adds almost nothing over process data

Adding in-line inspection results to the defect classifier changes AUC by −0.002. Because the inspection signals are downstream consequences of the same process excursions, they carry little independent information. The operational implication is favourable: process data is sufficient, and it is available hours earlier.

## 9.3  Heavy e-trucks are near carbon parity on India's grid

Before the renewable depot charging assumption is applied, medium and heavy electric trucks running on India's coal-heavy grid sit close to well-to-wheel parity with diesel. This is a genuine physical result, not a modelling artefact, and it argues that grid decarbonisation and captive solar at depots are prerequisites for heavy-fleet electrification to deliver its expected carbon benefit.

# 10. Known limitations

Stated directly, because a reviewer will find them.

- Manufacturing data is synthetic. No open cell-manufacturing defect corpus exists. The QMS pipeline is real; the batches are generated.

- B0018 misses its baseline. The degradation model underperforms persistence on one of four real cells. The square-root-plus-linear form cannot represent every capacity-knee trajectory.

- No expert baseline for readiness scoring. The brief's evaluation focus asks for fleet electrification readiness quality against an expert baseline. We report the scoring methodology but have not benchmarked it against domain-expert judgement.

- No carbon validation against measured emissions. Carbon figures are modelled from published factors and are not reconciled against actual metered emissions.

- Supply chain lead time is modelled, not measured. Disruption signals and procurement lead times are represented but detection lead time is not quantified as a metric.

- In-memory data store. The prototype seeds deterministically at startup with no database. Every service reads through one interface, so this is a swap rather than a rewrite, but it is not production persistence.

# 11. Deliverables

| Deliverable | Status |
| --- | --- |
| Working prototype | Complete — 41 endpoints, 8 pages, verified end to end |
| Architecture diagram | Complete — docs/architecture.html |
| Project report | This document |
| Demo script | Complete — DEMO.md |
| Presentation deck | Outstanding |
| Demo video | Outstanding |

# 12. Running the prototype

## Backend

cd backend  →  pip install -r requirements.txt  →  uvicorn main:app --reload --port 8000

## Frontend

cd frontend  →  npm install  →  npm run dev

The application is then served at http://localhost:3000, with the interactive API explorer at http://localhost:8000/docs.

Two operational notes uvicorn resolves main:app relative to the working directory. It must be run from inside backend/ — running it from the repository root fails with "Could not import module main", which reads like a code error but is a path error. GEMINI_API_KEY must be set before the backend starts, in the same shell. Without it the orchestrator runs in deterministic mode: it still plans, calls tools and answers correctly using template synthesis, but the Agent Console badge will read DETERMINISTIC rather than LLM.

# 13. Technology stack

| Layer | Technology |
| --- | --- |
| Backend | FastAPI, Pydantic, Uvicorn |
| Machine learning | scikit-learn (logistic regression, metrics), NumPy (least-squares fitting), pandas |
| Frontend | Next.js 16, React 19, Recharts, CSS Modules |
| Agents | Custom orchestrator — planner, specialists, synthesiser — over a 10-tool registry; Google Gemini optional |
| Data | NASA PCoE ageing dataset (CC0); deterministic in-memory seeded store |

# 14. Roadmap

Capabilities identified as valuable but deliberately out of scope for the prototype:

- Physics-informed machine learning via PyBaMM — learning residuals against a Single Particle Model rather than fitting an empirical curve.

- Electric vehicle routing (EVRP) with chance constraints, using OSRM and OR-Tools over real road networks.

- Computer vision for weld-seam and electrode inspection using 3D laser profile data.

- Live geopolitical signal ingestion for supply chain risk, replacing the modelled alert stream.

- Integration with the Battery Pack Aadhaar central server once the scheme is finalised.

- Persistent storage and multi-tenant deployment.

All figures in this document were produced by live endpoints and verified at time of writing. Data regenerates deterministically from a fixed seed on backend restart; figures will shift if model code changes.